# Flash update steps

Vista support requires that the BIOS on the SATA PCIe card be updated (flashed) before the driver can be installed.  You’ll find all the necessary files to update the card’s BIOS.  Please contact SIIG Technical support if you have any questions.

The flash update utility works in DOS only (not Windows DOS mode).  Use a DOS boot disk or a Windows startup disk (95b/98/ME) to boot the computer.

Do not have any devices connected to the SIIG controller during the flash update.

- Copy updflash.exe and the BIOS file to a floppy disk.

- Boot the system with the DOS or Windows startup floppy disk.

- Insert the floppy that contains the bios and flash utility.  Type updflash b7313.bin, then hit the Enter key.  (The entire name, including extension, of the bios file must be used)

- On screen will display: UPD Flash V2.0, SiI xxx version 2, Loading Bios…, Verifying…, Bios is loaded, then back to the A prompt. The entire process takes 30-45 seconds.

- Remove the floppy then restart the computer.  Check the bios version.