Windows XP Driver Installation

- After extracting the Zip file, double click on PL2303_Prolific_DriverInstaller_10311.exe to start the driver installation.

- At Welcome screen, click Next.

- At InstallShield Wizard Complete, click Finish to complete driver installation.

- Change the Matrix’s SW Main DIP switch position to USB port (Off, Off, Off, ON).  See page 5 in the Quick Installation Guide for more information.

- Connect the Matrix’s USB port to the computer’s USB port using a USB Type A (Male) to Type B (Male) cable (USB cable not included).

- Power on the Matrix and the USB serial driver will install automatically.

Installing the Matrix Control Software

If you have not installed the Matrix Control software into your computer please install it now, use the directions below or the Quick Installation Guide.  Otherwise, skip this section and go directly to Software Control through USB Port to configure the Matrix Control software.

- Insert the included installation CD into the CD-ROM drive of your computer.

- Open Windows Explorer and double click on the CD-ROM drive, then double click Setup to start the software installation.

- Follow the onscreen prompts to complete the software installation.

- Start the HDMI Matrix Control software utility and the Control Software Menu, similar to below, will open.

Software Control through USB Port

- Start the HDMI Matrix Control software utility and the Control Software Menu, similar to below, will open.  Click the Scan icon and go to step 2.

- Click Scan again to display the status of all COM ports.

- Select the USB serial port connected to the Matrix and set device ID to 255 for all devices.  Note: Only device ID 255 should be used.

- Click OK to save the settings and return to the main menu.

- The Matrix Control software is ready for use.

For additional features, setup and operating instructions, use the instructions found in included Quick Installation Guide.

Uninstalling the USB driver

Double click on PL2303_Prolific_DriverInstaller_10311.exe and follow the directions to uninstall the driver.