RS-422/485 Terminal Block Wiring and DIP Switch Settings

For USB To RS-422/485 Converter

Terminator (DIP Switches)

The purpose of the Terminator (DIP Switches) is for compensating signal attenuation in a long distance connection at RS-485/422.  To activate the signal compensation, please set the dip switch 1 & 2 to “ON” position.  To disable signal compensation, please set dip switch 1 & 2 to “OFF” position.

□RS-422 Pin Assignment

The pin assignment scheme for a 4-pin RS-422 is given below.

1  2  3  4

RS-422:  PIN 1: T+    PIN 2: T-   PIN 3: R+   PIN 4: R-

RS-485:  PIN 1: D+    PIN 2: D-

- □RS-422 Wiring Diagram

Serial Device                USB To 422/485 Converter

R-                              2  T-

R+                             1  T+

T-                              4  R-

T+                             3  R+

- □RS-485 Wiring Diagram

Serial Device               USB To 422/485 Converter

D-                              2  D-

D+                              1  D+